FreeSpeak II Bluetooth License Ticket

Permanent License(s)

Quantity	0	FSII-BP19-BT-UPG	Bluetooth Activation

Dear Customer,

Thank you for purchasing a Clear-Com License. Please find your license details below.

Ticket ID: {LICENSE TICKET FROM KeyGen.sh}

How to install and activate your Bluetooth licenses

Download Clear-Com Power Tools from the Clear-Com partner portal.

- https://www.clearcom.com/Portal-Login

Install Power tools on a Windows PC.

Open Power Tools and navigate to the Licensing Tab

Type/Paste License Code Here

After entering license code, the name and quantity of licenses will show in the license field as valid.

Plug FreeSpeak II Beltpack (s) (you can use USB hub to plug in up to 8 Beltpacks)

- (Require Micro USB Cable that supports data - FSII BP ships with USB cable)

- Attached beltpacks will show in Bluetooth field.

If the beltpack firmware needs to be updated, then this field will appear, and you will need to update before doing Bluetooth update.

Push the update firmware and status will appear in blue and in the window at the bottom.

When beltpack firmware is update, you can then start the Bluetooth update.

For more information, please refer to the User Documentation.

Clear-Com Contacts

| Americas and Asia-Pacific Headquarters California, United States Tel: +1.510.337.6600 Email:CustomerServicesUS@clearcom.com | Europe, Middle East, and Africa Headquarters Cambridge, United Kingdom Tel: +44 1223 815000 Email:SalesSupportEMEA@clearcom.com |
| --- | --- |
| China Office Beijing Representative Office Beijing, P.R.China Tel: +8610 5811360/65815577 |  |