# V-Series Iris Main Board Production Plan

Contents

V-Series Iris Main Board Production Plan	1

The physical environment	2

The production workbench	2

Prime Station	2

Golden Units	3

Host (Prime Station)	4

The Directory’s structure and file location	5

Programming Display Modules	5

Programming the FPGA	5

Programming Main Boards	5

Data Flow	6

Engineering One Time Configuration	6

Production Administrator one-time Configuration	7

Programming the Main Board	7

Automation	8

Testing	8

Report	9

## The physical environment

In this section a description of the physical environment that will be provided. The physical environment details the hardware required for production of V-Series Iris main boards and how the hardware components connect to each other.

### The production workbench

After assembly of the VSI hardware, the units will arrive at an area designated for programming by production. The term workbench is used to describe the set of equipment required to perform the production procedure in this document.

The required equipment on the workbench consists of:

- 1 The Prime-Station (see section x)

- 1 mPower 8 ports Power bar (give model num)

  - Two ports per target Board, one to control the Power Supply, the other to control the sysboot jumper.

- 1 Ethernet Switch (suggest model)

  - Used to communicate with the Super-Station and with all the components.

- 4 Relay controllers

  - Control the sysboot pins.

- 4 usb-uart cables ( FTDI )

  - One for each Main Board

- 1 usb hub (model num)

- 4 dedicated regions for docking main boards. Each equipped with:

  - Wire harness consisting of Ethernet, power, sysboot control and serial cables

  - V32 Golden Display module (see section x)

### Prime Station

The computer used to control the programming procedure for VSI Main Boards is called the Prime Station. It is a POTS (Plain Off The Shelf) system running Windows with software provided by development explicitly for the purpose of programming VSI Main Boards. Beyond just software, the Prime station also consists of cables and connectors that must be set up in a specific way.

** alternate text**

The Prime station is the computer that will host all the software needed to program the V-Series Iris panels. More than just containing software, it is a reference platform where the hardware is configured exactly according to this document (see section x on the physical environment).

**

The cables should be set up as follows: TODO some more description

### Golden Units

A golden unit is a reference piece of hardware in known working condition.

There must be a golden v32 Display board in each of the dedicated main board docking areas. When a new main board is installed on the workbench, the operator must connect the corresponding golden display board to it.

Todo

The Prime-Station can program four Target Boards at a time. Below is the list of equipment needed:

- 1 The Prime-Station

  - Run the C2L3 software

- 1 mPower 8 ports Power bar

  - Two ports per target Board, one to control the Power Supply, the other to control the sysboot jumper.

- 1 Ethernet Switch

  - Used to communicate with the Super-Station and with all the components.

- 4 Relay controllers

  - Control the sysboot pins.

- 4 usb-uart cables

  - Console ports, one for each Target Board

- 1 usb hub

- 4 Sets of Target Boards, each set of which consist of:

  - One V32 golden units

  - One golden expansion panel

  - The Main Board to be loaded and tested.

## Prime Station Configuration

The Prime station introduced in section x will need to run the latest software released by development. A zip file will be provided with all the required software and some instructions for setting up said software. Upgrade instructions will also be provided if necessary.

Using the prime station an operator will be able to efficiently program panels with an environment similar (if not the same) that is used in development so that it will be easy for collaboration if necessary.

### The Directory’s structure and file location

Files are organized by project under the   c:\ drive.

The Project Path (PROJ_PATH):

C:\ PROJ_PATH \

Example: For Pluto project path is “c:\Pluto\”

The Development Path (dev):

C:\ Pluto \dev\config

C:\ Pluto \dev\bin

The Production Path (prod):

C:\ Pluto \prod\config

C:\ Pluto \prod\bin

The C2L3 software is located at:

C:\CCLowLevelLoader\ CC_Serial_Tool.exe

## Programming Display Modules

The display modules will be programmed using a procedure described in the xyz document.

## Programming the FPGA

** To be defined **

There are 2 possible ways to accomplish FPGA Programming

- Programming using Qartus on the Prime station

  - Nothing to be developed

  - Same as Jupiter/Saturn

- JTAG from Sitara

  - Completely automated

  - Still needs to be completed

## Programming Main Boards

This section details the procedure for programming the main V-Series Iris boards using the Prime Station. Up to 4 main boards can be programmed in parallel.

## Data Flow

Before production begins some data needs to be prepared and given to the Prime Station. Additionally the results of programming needs to be recorded and stored. This section details the inputs and outputs of the production process.

### Production Administrator one-time Configuration

#### Mac Address control

The C2L3 tool allows the administrator to set a range of MAC address used exclusively by the Prime Station.

A range of MAC addresses must be allocated to the VSI project before production begins. Once this range in known, C2L3 can be configured to use it. C2L3 will own this block of addresses and eliminates the need to track MAC addresses with other tools.

Below is the Production MAC address settings screen from C2L3. It must be configured by an administrator before production begins. It is not required to be adjusted after initially configured unless a new block is desired. C2L3 will take care of logging which boards have been programmed with wich serial numbers and some additional information.

- MAC Start:

  - The first MAC address in the range assigned to the project.

- MAC End

  - The last MAC address in the range (inclusive).

- Total

  - Caclulated by C2L3

- MAC Used

  - Caclulated by C2L3

- MAC Available

  - Caclulated by C2L3

- # of MAC for CPU0

  - Use 1 for VSI

- # of MAC for CPU1

  - Use 0 for VSI

- # of MAC for FPGA

  - Use 1 for VSI

Prime Station Configuration

C2L3 allows the administrator (with help from Enginner) to configure the initial settings of the Prime Station.

C2L3 will be delivered with default settings for the production of VSI.  These settings must be verified by Development before production begins and should not need adjusting throughout the project.

### Programming the Main Board

To start the C2L3 in production mode for Pluto project:

Launch C2L3 using the provided shortcut.

C:\CCLowLevelLoader\ CC_Serial_Tool.exe Pluto

Each time the software is executed, it will prompt for the Employee number. This is only requested when C2L3 is started. It is suggested that the operator exits C2L3 at the end of their shift. The employee number, date, time and other info will be saved in the board history databases on the Prime Station and on the Main board itself.

The screen shown below is the main window of C2L3. Its job is to program main boards in a few simple steps. The Prime station will be equipped with a optical bar code scanner that will be used to tell C2L3 which board is located in which docking region.

To program a main board, connect all the standard cables from the wiring harness to the VSI board as shown in section x. The Golden Display Module should also be connected at this time.

Follow these steps to complete programming of the VSI main board:

- Note to update screenshot

- Using the barcode scanner, scan the Dock barcode next to the first main board connected in the dock region.

- Scan the barcode on the middle of the main board PCB. Be sure to scan the one corresponding to the barcode in the previous step.

- Scan the Dock barcode of the same dock position again to confirm

C2L3 will then start programming the boards.

## Automation

Several of the steps in the production procedure for V-Series Iris panels can be optimized for the least amount of human interaction. This helps to speed up the process and reduce operator error. This section will detail such optimizations.

The Production Steps and Timeout values are new concept introduced to allow several programming steps to be automated. C2L3 will send the commands to the board being programmed and wait for results up to the defined timeout settings. The values shipped with C2L3 should not be changed without input from the developers but provide an opportunity for tuning.

## Testing

After the panel hardware is programmed and assembled, production tests can be run to verify the hardware functionality. This section will detail the V-Series Iris production tests.

TODO

## Report

This section details the report that is generated after programming a main board.

Todo ask what kind of data / format they want and at what stage.